Liz Hetherington

Localization Exercise

10 September 2007

Beam Reach 071

For my research I will need to localize vocalizations and pin point them to a specific  individual in order to distinguish acoustics differences among sexes and age group. In order to localize vocalizations we need to use the program Ishmael 1.0.  When we were on the Gato Verde we did a localizing experiment with the Gatito. The Gatito was sent out with an underwater speaker and we conducted a playback experiment. We played both hammer sounds and killer whale calls. The Gato Verde drove by the Gatito with the hydrophone array deployed and we recorded the sounds from the speaker. The array records on 4 channels which recorded to one sound file that we can then analyze. We also used the range finder and a protractor to get the distance and the bearing on the Gatito. The recordings were then put into the hard drive and analyze them using Ishmael.

When we first open the Ishmael program, I clicked on open file and chose the 070904_kwpb_s1_16bit.wav. It must be a 16 bit wav or else Ishamel cannot read it. After opening the file, I clicked on Localization, then Loc option. I changed the speed of sound to 1485 m/s. I then selected the phone array file that I created in notepad earlier. Due to this, the coordinates that Ishmael will give me are accurate because they are in regards to the distance that the hydrophone array is from the boat. I then selected a call at 23 seconds, and clicked on hyperbolic loc. When I did this, two screens came up: one was the Cross-correlation and the other was the location. The cross correlation is always between a pair of channels. When the correlation is the highest, the match is the best (the spike). The higher the spike, the better the correlation is. Gives you a sense on how good the cross correlation is which tells you how much you can trust the location. If there are really little spikes it means that the channels aren’t as clear/good. The other screen that came up was the Location which gave me the x and y coordinates.

I then took those coordinates and put them into an excel spreadsheet to figure out the distance and the bearing. The distance can be calculated by using the Pythagorean Theorem, using x² + y² = c². The c value is the distance, and from this value we can obtain the Bearing in radians. The bearing is an angle that is defined by sin=opposite/hypotenuse. The opposite value in the right triangle is the x value, and the hypotenuse value is the c value. Dividing x/c gives us the bearing in radians, which then needs to be converted from radians into degrees. After the conversion we then have the bearing and the distance which is the two values needed to test the accuracy of our localization. We take these numbers and compare them to the data we collected on the boat using the protractor and the range finder.

| Time (s) | X | Y | Distance | Bearing (radians) | Bearing (Degrees) |  | Distance (m) | Bearing (degrees) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 (124) | -80.9347 | -127.624 | 151.1235 | -0.565162893 | -32.38144848 | Add 180- 148 | 73 | 130 |
| 12 (1340 | -99.4094 | -164.964 | 192.6015 | -0.542338383 | -31.07370039 | 149 | 83 | 145 |