Elise Chapman

11 September 2007

Localization Exercise

The ability to localize the source of a killer whale vocalization provides the bearing and distance from the origin, port aft of Gato Verde (GV), to the killer whale of interest.  Such information can be used to determine the level of source amplitudes, is an important component of my research project.  As an exercise in localization we deployed the hydrophone array from the GV while the underwater speaker was deployed from the dinghy.  The program Ishmael is able to take the four-channel hydrophone recording and localize the source of the sound.  .  As we recorded sound received by the hydrophone the bearing of Gatito relative to GV was determined using a protractor and Gatito’s distance was determined with a rangefinder.

To localize a recorded sound we first opened the recording of interest in Ishmael and created a log file for the results.  Under the “Localize” Tab “loc options…” was selected to bring up Localization parameters screen.  On this screen the “Basic Info” tab is used to prompt Ishmael to find the array specifications and to set the speed of sound.

We are using hyperbolic localization so the “Hyperbolic” tab was selected from the same “Localization parameters” screen to set the map area where the localizations will be graphically represented.  On the spectrogram the call of interest was selected at time 42 seconds.  To select the call of interest the cursor was dragged over the call area on the spectrogram.  This enables the localization buttons on the second line of tabs at the top of the Ishmael screen.  The “hyperbolic loc” button was then pushed to run the localization.

Ishmael outputs two screens and a log file once it has completed the localization.  The first is the cross correlation comparing the signals from all combinations of hydrophones used in the localizations.  Because clear peaks appeared in all the cross correlations it was reasonable to accept the localized position of the source.  The second is the map of the localization.  The four red dots represent the hydrophone array and the yellow dot is the source location.  In the heading of the later window the localized position of the source is represented by coordinates in the x and y planes.  The third screen also includes the coordinates for the localized source and has the time delay between the six hydrophone pairs.

Screen 1				   Screen 2

Screen 3

Using the x and y coordinates from Ishmael as input for the Pythagorean theorem, the distance from the boat to the source of the sound can be determined.  Given the properties of a triangle the bearing can be determined by taking the arcsin of the x value divided by the hypotenuse (distance).  In some cases (the first and second calls localized) the distance from Ishmael was comparable to the measured distance.  In the last case it was not a good localization as it grossly overestimated the distance.  For the last case it did have an accurate bearing value but the first two calls were not accurate in their bearings.