Kenna Lehmann

Localization Exercise

September 11, 2007

On the 3rd of September 2007, the Beam Reach JaMi team, aboard the Gato Verde, gathered sound recordings for a localization exercise.  The sound files were recorded from playbacks being emitted from an underwater speaker suspended from a small vessel (the Gatito).  The small vessel remained stationary as the Gato Verde passed by with a microphone array dragging behind.  The sound files were recorded onto solid state recorders in four different channels at a sampling rate 44100 and a 24 bit rate.

As the sound files were being recorded, students on the Gato Verde took measurements on the distance and bearing of the Gatito.

Later, Jason Wood broke the sound files into one minute increments and changed them over to 16 bit so that Ishmael could analyze them.

Back on land we used Ishmael to localize the sounds that were emitted from the Gatito's underwater speaker.  Ishmael requires three things to analyze the sound files: The placement of the hydrophones on a grid, the speed of sound through the medium, and the sound file.  We first entered the distances of the hydrophone from the origin into excel and then saved the file as an .arr file.  The port aft corner of the Gato Verde was designated as the map origin with the hydrophones extending down along the y-axis.  In Ishmael we inputted the speed of sound and where the .arr file was located.  Then, we highlighted one call and localized it using the hyperbolic function.

Ishmael then outputs a localization map and graphs of cross correlations to illustrate how confident the computer is in the given location of the call.  If the cross correlation graphs show a high central spike without much deviation from that then the location is likely accurate.  We then looked at the accurate localizations and used the coordinates and Pythagorean’s theorem to determine the distance of the call from the boat.  Trigonometry was also utilized to determine the animals bearing from the Gato Verde.

With the data from Ishmael giving the location at which the call was made, we can now look at the data recorded during sound recording to match the localized call with the distance and bearing where a known individual was seen.

| Ishmael x | Ishmael y | Ishmael Distance | Ishmael Angle Radians | Ishmael Angle Degrees | Recorded Distance | Recorded Bearing |
| --- | --- | --- | --- | --- | --- | --- |
| -53.0882 | 22.7844 | 57.77097768 | 2.736186742 | 156.7719523 | 85m | 55 |
| -40.0804 | -49.9794 | 64.06542662 | 2.246718102 | 128.727465 | 83m | 145 |

The first Ishmael output angle is taken from the back of the boat at 0 degrees.  180-157 gives 23 which is close to the recorded 55 degrees.